# TASK 4: Ethernet, Internet Communication, and Wireless Technologies

| Student Name: | Tarek Ahmed Umar |
| --- | --- |
| Unit Title: | Networking |
| Unit Code: | WNI518 |
| Assessment Criteria: | 4.1, 4.2, 4.3 |
| Assessor Name: | Shafaat Shah |
| Due Date: | 06/03/2026 |

Table of Contents

Task 4.1 – Communication Across a Local Ethernet Network	1

Task 4.2 – Internet Communication Between Hosts	2

Task 4.3 – Evaluation of Three Wireless Technologies	3

References	4

## Task 4.1 - Communication Across a Local Ethernet Network

The IEEE 802.3 is a standard that regulates the operation of an Ethernet network at OSI layers 1 and 2, which are the physical and data link layers, respectively. The process of encapsulation of an application payload is performed downwards through the OSI layers during transmission of a network packet. The process involves segmenting a network packet into smaller segments during transport layer operation, followed by IP packet formation during network layer operation, and finally, formation of an Ethernet frame during data link layer operation, after which the signal is transmitted over the physical media (Fluke Networks, 2022).

The various components of an Ethernet frame include a destination MAC address, a source MAC address, a payload of a maximum of 1500 bytes, an EtherType that identifies the encapsulated protocol, and a Frame Check Sequence that is used for error detection. Each network interface card is assigned a 48-bit MAC address that is permanently assigned and is only used for delivering network packets within a network segment. An ARP broadcast is sent to all devices on a network segment in order to identify the IP address of a destination device and its corresponding MAC address in case a network device is unsure of the MAC address of a destination device. The ARP broadcast is sent to FF:FF:FF:FF:FF:FF (Forouzan, 2013).

Switches operate at Layer 2. A table is created with the source MAC addresses of incoming frames and the ports they came in on. A single isolated collision domain is created for each port, with full duplex communication possible on all lines at once through the table to forward frames to the correct port based on the destination MAC address. Half duplex CSMA/CD must be used by devices to detect collisions in a single shared collision domain created by a hub, which operates at Layer 1 and simply replays every incoming signal on every port (Network Academy, 2020).

Switches eliminate a collision domain for each device connected to them; however, a device connected to a switch remains in the same broadcast domain. This is due to the fact that every device will receive a frame sent to the broadcast MAC address. This can hinder performance with a growing number of devices. By routing IP addresses outside the local area network, such as when a broadband router forwards traffic from the office network to the internet, routers function at Layer 3, separating broadcast domains between network segments (Tanenbaum and Wetherall, 2011).

## Task 4.2 - Internet Communication Between Hosts

TCP/IP is a set of protocols used for communication between different networks that make up the Internet. It has four levels: Network Access, Internet, Transport, and Application. In order for identification and routing purposes, every device connected to the Internet requires a unique IP address. As a directory service for the Internet, the Domain Name System (DNS) translates domain names like www.google.com into IP addresses necessary for routing (TechTarget, 2024).

IPv4 has 32-bit addresses with a maximum capacity of 4.3 billion addresses. However, due to the increase in the number of internet-connected devices worldwide in 2011 , IANA has already exhausted this capacity. In order to address this limitation, IPv6 has 128-bit addresses with a capacity of more than 340 undecillion addresses. It also provides other features such as stateless address autoconfiguration and routing efficiency. Dual stack is a feature that allows a smooth transition between IPv4 and IPv6 without affecting existing IPv4 infrastructure (Forouzan, 2013).

Packet switching is used for the transmission of data over the Internet. Application data is divided into packets, and each packet contains a payload segment, a source IP address, and a destination IP address. In order for the routers to forward the packets individually towards the destination, the routers accept the individual packets and refer to their routing tables, which are updated using routing protocols such as OSPF within the organization and BGP between the ISPs. At the receiving end, the individual packets are reassembled in the correct order after traveling through the network in different paths (Tanenbaum and Wetherall, 2011 ).

TCP is used for reliable and connection-oriented data transfer at the Transport layer. TCP sends the segments with a number and re-sends them in the event of loss or corruption, ensuring reliable and ordered delivery. TCP establishes a connection with the help of a three-way hand-shaking protocol, where the client sends a SYN segment, the server sends a SYN-ACK segment, and the client sends an ACK segment. However, in the case of applications such as VoIP and video streaming, where the connection needs to be real-time, the guarantee of reliable delivery is not required, and in such a scenario, the UDP protocol is used, which is connectionless (TechTarget, 2024).

## Task 4.3 - Evaluation of Three Wireless Technologies

Three wireless technologies: Wi-Fi (IEEE 802.11), Bluetooth (IEEE 802.15.1), and Zigbee (IEEE 802.15.4) are designed for specific applications. The main characteristics of the wireless technologies are compared in the table below.

| Feature | Wi-Fi (IEEE 802.11) | Bluetooth (802.15.1) | Zigbee (IEEE 802.15.4) |
| --- | --- | --- | --- |
| Frequency | 2.4 GHz / 5 GHz / 6 GHz | 2.4 GHz | 2.4 GHz |
| Max Data Rate | Up to 9.6 Gbps (Wi-Fi 6) | 1–3 Mbps (BLE: 1–2 Mbps) | 20–250 Kbps |
| Typical Range | 30–100 m indoors | 10–100 m (class-dependent) | Up to 100 m (mesh extends further) |
| Power Consumption | High | Very Low (BLE) | Extremely Low |
| Network Topology | Star (access point) | Point-to-point / piconet | Mesh / star / tree |
| Primary Use Cases | Home/office networking, streaming, cloud apps | Headphones, keyboards, wearables, medical devices | Smart home IoT, industrial sensors, building automation |

Wi-Fi is the standard for cloud applications, streaming of high-definition movies, video conferencing, and home or business internet connectivity because of its fastest data rate of transmission. It also supports WPA3 for encryption. The major drawback of Wi-Fi is its high- power consumption, which makes it unsuitable for small IoT devices needing battery power for long periods of time without recharging. Wi-Fi 6 (802.11ax ) is used in the TP-Link Archer AX55 router for wireless devices in Task 1 network connectivity (MokoSmart, 2025).

Bluetooth, particularly BLE, is optimized for PANs for short-range communication with extremely low power consumption, making it suitable for wireless communication between smartphone peripherals such as wireless headphones, computer peripherals such as wireless keyboards, fitness devices, and medical devices. It can be powered for months using a coin cell battery. The limitations of Bluetooth are its short range without the use of repeaters for increasing the range and its slow data rate compared to Wi-Fi for wireless communication. AES- 128 encryption is sufficient for secure communication between devices (MokoSmart, 2025 ).

Zigbee wireless technology is optimized for IoT devices for wireless communication using small batteries for IoT devices needing long periods of battery life because of its mesh topology, which enables devices to route data through intermediate devices for extended communication range and self-recovery from device failure in the network. It is suitable for industrial automation, home automation systems, which include smart home devices such as smart light bulbs, smart home thermostats, smart door locks, etc. The limitations of Zigbee wireless technology are its inability to handle large amounts of data because of its poor throughput of 20-250 Kbps (Cdebyte,  2024).

## References

Cdebyte (2024) Comparison of Bluetooth vs Zigbee vs WiFi. Available at: https://www.cdebyte.com/news/846 (Accessed: 24 February 2026).

Fluke Networks (2022) 101 Series Ethernet: Back to Basics. Available at: https://www.flukenetworks.com/blog/cabling-chronicles/101-series-ethernet-back-basics (Accessed: 24 February 2026).

Forouzan, B.A. (2013) Data Communications and Networking. 5th edn. New York: McGraw-Hill.

MokoSmart (2025) Bluetooth vs WiFi vs Zigbee: Which Wireless Technology is Better? Available at: https://www.mokosmart.com/bluetooth-vs-wifi-vs-zigbee-which-is-better/ (Accessed: 24 February 2026).

Network Academy (2020) Collision Domains. Available at: https://www.networkacademy.io/ccna/ethernet/collision-domains (Accessed: 24 February 2026).

Tanenbaum, A.S. and Wetherall, D. (2011) Computer Networks. 5th edn. Harlow: Pearson Education.

TechTarget (2024) What is TCP/IP and How Does it Work? Available at: https://www.techtarget.com/searchnetworking/definition/TCP-IP (Accessed: 24 February 2026).